Review – 4

Team –A11

USN: 02FE21BEC058	–	POOJA NANDAGAON

USN: 02FE21BEC059	–	PRAJWAL HALGI

USN: 02FE21BEC061	–	PRAJWAL KAMBLE

USN: 02FE21BEC068	–      RAKSHITA SHIVAPOOJI

Topic - Write a C program with two tasks and resource LCD

Under The Guidance Of :        Guide Signature

Prof.	Swati M

CODE

#include <rtl.h>

#include<lpc213x.h>

#include<string.h>

#include "lcd.h"

////////////////////////

#define SLAVE_ADDR 78

#define MAX        12

#define AA         2

#define SI         3

#define STO        4

#define STA        5

#define I2EN       6

void data(unsigned int);

void LCD_init (void);

void LCD_Command(char);

void I2C_Init(void);

void delay_ms(int);

unsigned int i=0;

OS_TID tsk1, tsk2;

OS_SEM semaphore1;

/*----------------------------------------------------------------------------

*    Task 1 - High Priority - Active every 3 ticks

*---------------------------------------------------------------------------*/

__task void task1 (void) {

OS_RESULT ret;

char str[]  = {"HELLO"};

while (1) {

/* Pass control to other tasks for 3 OS ticks */

os_dly_wait(3);

/* Wait 1 ticks for the free semaphore */

ret = os_sem_wait (semaphore1, 1);

if (ret != OS_R_TMO) {

/* If there was no time-out the semaphore was aquired */

//printf ("Task 1\n");

LCD_String(str);

delay_ms(65000);delay_ms(65000);delay_ms(65000);

LCD_Command(0x01);

/* Return a token back to a semaphore */

os_sem_send (semaphore1);

}

}

}

/*----------------------------------------------------------------------------

*    Task 2 - Low Priority - looks for a free semaphore and uses the resource

*                            whenever it is available

*---------------------------------------------------------------------------*/

__task void task2 (void) {

char str[]  = {"WORLD"};

while (1)

{

/* Wait indefinetly for a free semaphore */

os_sem_wait (semaphore1, 0xFFFF);

/* OK, the serial interface is free now, use it. */

LCD_String(str);

delay_ms(65000);delay_ms(65000);delay_ms(65000);

LCD_Command(0x01);

/* Return a token back to a semaphore. */

os_sem_send (semaphore1);

}

}

/*----------------------------------------------------------------------------

*    Task 3 'init'

*---------------------------------------------------------------------------*/

__task void init (void) {

/* Initialize the Semaphore before the first use */

os_sem_init (semaphore1, 1);

/* Create an instance of task1 with priority 10 */

tsk1 = os_tsk_create (task1, 10);

/* Create an instance of task2 with default priority 1 */

tsk2 = os_tsk_create (task2, 0);

/* Delete the init task */

os_tsk_delete_self ();

}

/*----------------------------------------------------------------------------

*    Main: Initialize and start RTX Kernel

*---------------------------------------------------------------------------*/

int main (void)

{

IODIR0 = 0xf0ff00fc;

LCD_init();

/* Initialize RTX and start init   */

os_sys_init (init);

}

OUTPUT